# NETWORK FUNDAMENTALS AND BUILDING NETWORKS

# IP ROUTING AND IP PROTOCOLS

TASK1: Draw a simple network diagram showing three devices (like your phone, a friend's laptop, and a smart TV) connected through a router, and label the IP addresses for each device and the router's interfaces.

TASK2: Simulate manual IP routing by writing a step-by-step explanation of how a packet would travel from your laptop to a friend's computer in a different subnet, including which router(s) it passes through and how the routing decision is made.<br><br><em><strong>Hint:</strong> Use made-up IP addresses and show the routing table entries that would be checked at each hop.</em>

SCENARIO:

My laptop is on 192.168.1.10/24.

My friend's computer is on 192.168.2.20/24.

The router connects both networks.

STEPS:

- My laptop creates a packet for 192.168.2.20.

- The laptop checks its routing table.

- Since the destination is on a different subnet, the packet is sent to the default gateway (192.168.1.1).

- The router checks its routing table.

- The router finds that the 192.168.2.0/24 network is connected through its second interface.

- The router forwards the packet to the friend's computer.

- The friend's computer receives the packet and sends a reply back through the same router.

| Destination Network | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 192.168.1.0 | 255.255.255.0 | Local | LAN1 |
| 192.168.2.0 | 255.255.255.0 | Local | LAN2 |
| 0.0.0.0 | 0.0.0.0 | ISP | WAN |

TASK3: Create a routing table for a fictional Zomato-style delivery network with three branches in different cities, each on a different subnet. Specify the network address, subnet mask, next hop, and interface for each route.

| Branch | Network Address | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- | --- |
| Delhi | 10.0.1.0 | 255.255.255.0 | Direct | LAN1 |
| Mumbai | 10.0.2.0 | 255.255.255.0 | 192.168.100.2 | WAN |
| Indore | 10.0.3.0 | 255.255.255.0 | 192.168.100.3 | WAN |

TASK4: Compare static routing and dynamic routing protocols (like RIP or OSPF) by listing 3 pros and 3 cons of each, using examples from real-world apps or services you use (e.g., how a food delivery app might benefit from dynamic routing).

| Static Routing | Dynamic Routing |
| --- | --- |
| Pros |  |
| Easy to configure | Automatically updates routes |
| Uses less CPU and memory | Best for large networks |
| More secure because routes do not change automatically | Quickly finds a new route if a link fails |
| Cons |  |
| Must be updated manually | More complex to configure |
| Not suitable for large networks | Uses more CPU and memory |
| No automatic backup route | Can generate extra network traffic |

EXAMPLE:

- Static Routing: Suitable for a small office network.

- Dynamic Routing (OSPF/RIP): Better for a food delivery company like Zomato, where many branches need automatic route updates.

TASK5: Use ChatGPT or another AI tool to generate a sample configuration for setting up OSPF routing between two routers, then explain in your own words what each configuration line does.<br><br><em><strong>Hint:</strong> Paste the AI-generated config and annotate each part with a short description.</em>

SAMPLE CONFIGRATION:

Router(config)# router ospf 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# network 192.168.2.0 0.0.0.255 area 0

| Configuration Line | Description |
| --- | --- |
| router ospf 1 | Starts the OSPF routing process. |
| network 192.168.1.0 0.0.0.255 area 0 | Adds the 192.168.1.0 network to OSPF Area 0. |
| network 192.168.2.0 0.0.0.255 area 0 | Adds the 192.168.2.0 network to OSPF Area 0. |