# TROUBLESHOOTING AND HELPDESK

# Assignment DAY4

TASK1: Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process.

- Shut down the computer completely and disconnect the power cable/charger.

- Open the computer case/back panel using a screwdriver and locate the RAM slots on the motherboard.

- Release the RAM module by pushing the locking clips on both sides of the RAM slot outward.

- Remove the RAM carefully by holding it from the edges and avoiding touching the gold contacts.

- Inspect the RAM and slot for dust, dirt, physical damage, or bent components. No damage was observed during the inspection.

- Re-insert the RAM module by aligning the notch on the RAM with the notch in the slot and pressing it down firmly until the clips lock into place.

- Close the case/back panel, reconnect the power, and switch on the computer.

- Check the result: The computer should start normally and recognize the installed RAM.

Issues observed: No physical damage or unusual marks were observed. The RAM was slightly loose before reseating, and after firmly inserting it, the module was securely locked in the slot.

TASK2: .Create a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix.<br><br><em><strong>Hint:</strong> Think about display issues, driver checks, and physical connections.</em>

| Step | Action |
| --- | --- |
| 1 | Check if the monitor displays a blank screen, artifacts, or "No Signal." |
| 2 | Verify that the monitor cable is securely connected to the graphics card. |
| 3 | Remove and firmly reinsert the GPU into the PCIe slot. |
| 4 | Check that all GPU power connectors are properly connected. |
| 5 | Update or reinstall the graphics driver. |
| 6 | Test the GPU in another PC or test another GPU in the same PC. |
| 7 | Confirm that the display works correctly after the fix. |

TASK3: Simulate a storage issue by unplugging and replugging a SATA or NVMe drive (SSD/HDD) on a test system, then describe the error messages or BIOS indicators you see before and after reconnecting.

Procedure:

- Shut down the test computer completely and disconnect the power cable.

- Open the PC case and locate the SATA HDD/SSD or NVMe SSD.

- Carefully disconnect the SATA data/power cable (or remove the NVMe drive) to simulate a storage connection problem.

- Turn the computer on and enter the BIOS/UEFI settings by pressing a key such as Delete or F2.

- Before reconnecting: The storage drive does not appear in the BIOS storage/boot-device list. If the system tries to boot, it may show “No Boot Device Found,” “Operating System Not Found,” or a similar message.

- Turn the computer off again and reconnect the SATA cables firmly (or reinstall the NVMe drive properly).

- Start the computer and check the BIOS again. After reconnecting: the drive appears correctly in the BIOS/UEFI storage or boot-device list.

- If it is the system drive, the computer should then be able to boot into the operating system normally.

TASK4: Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps you would perform for that component.

Component I would check first: RAM

Reasoning: I would check the RAM first because faulty, loose, or incompatible RAM can cause system instability, crashes, and random restarts. It is also relatively easy to inspect and reseat compared with diagnosing the GPU or storage.

### Diagnostic Steps for RAM

- Shut down the PC and unplug the power cable.

- Open the cabinet and locate the RAM modules on the motherboard.

- Check the RAM physically for dust, damage, or improper seating.

- Reseat the RAM: Remove the RAM carefully and insert it back firmly until the side clips lock.

- Test one RAM stick at a time: If there are multiple modules, remove all except one and start the PC. Repeat with each stick to identify a potentially faulty module.

- Try different RAM slots: If the problem continues, test the working RAM module in another compatible slot.

- Check the result: If the random restarts stop with one particular RAM stick or slot, that component/slot may be faulty.